  - The general connection diagram of TIG160S/200S/180A

1.3 The diagram of TIG160S/200S/180A’s top PCB PM-01-A3, center PCB PZ-23-A0, and control module PK-02-A1

Troubleshooting of TIG160S/200S/180A

| Trouble | Analysis | Solution |
| --- | --- | --- |
| 1. Turn on the machine, the power indicator is not on, the fan doesn’t work, press the welding torch switch but no response. | - The input voltage is abnormal. - The input cable is not in good connection. - The power supply switch may be damaged. | - Check if the input voltage is AC 220V. - Check. - Replace the power supply switch if it’s damaged. |
| 2. Turn on the machine, the power indicator is on, but the fan doesn’t work or stops after few turns, press the welding torch switch but no response. | - The connecting cable of CP3/CP4 from the power supply switch to bottom PCB PZ-23-A0 is in loose connection. - The input voltage is too high, which leads to over-voltage protection. - The prime relay RELAY3 doesn’t close well; the value of resistor RT2/RV2/RV3/RV4 increases. - The auxiliary power supply part on top PCB PM-01-A3 is damaged. | - Check. - Check the input voltage. - Check and replace if necessary. - Check with a multi-meter if resistor R22/R24, MOSFET VT2, diode D6/D7/D8/D9/D10, zener diode Z3/Z4, OC U2, audion Q1 on top PCB PM-01-A3 is OK.. |
| 3. Turn on the machine, the power indicator is on, the fan works, the malfunction LED is not on, press the welding torch switch but no response. | - Loose connection inside the machine exists. - Some parts on top PCB PM-01-A3 or on control module PK-02-A1 are damaged. - The rectifying bridge matching the socket CON5/CON8 is disconnected. | - Check if the manual switch board PH-10-A1is OK. Short-circuit it with the socket CON10 on bottom PCB PZ-23-A0 to check. - Check with a multi-meter if the MOSFET VT7/VT8/VT9/VT10, manostat U3 on top PCB, or diode D1/D2/D3, audion Q1/Q2/Q3/Q5/ Q6/Q7/Q8/Q9, zener diode Z1/Z2/Z3/Z4, chip U1 is damaged. Check if the 16th pin of U1 is 5V. If not, replace U1 because it’s damaged. - Check if the voltage at both ends of socket CON3-CON4 on top PM-01-A3 is DC310V. |
| 4. Turn on the machine and it appears normal, press the welding torch and there is gas out, the malfunction LED is not on, no HF. | - The connecting cable matching socket CON7 on top PCB PM-01-A3 with socket CON3 on bottom PCB PZ-23-A0 is in loose connection. - High voltage silicon granule D2/D3/D4/D5 or high voltage output capacitor C14/C15 is damaged. - CP1/CP2 is disconnected with the bottom PCB. - The connecting cable matching socket CON2 on center PCB with socket CON2 on bottom PCB is in loose connection, or HF relay RELAY1 is damaged. - The HF discharge nozzle on bottom PCB has conglutination, excessive clearance or serious oxidation problem. - The ARC/TIG conversion switch on the panel of TIG180A is damaged. | - Check the output terminal of the machine. Disconnect the HF socket CON7 on top PCB, press the welding torch switch, and then check with a multi-meter if there is no-load voltage output. If no, deal with it according to trouble 3.b.c; if there is, deal with it according to trouble 4. - Check. - Check. - Check and replace it if necessary. - Adjust or replace it if necessary. - Check and replace it if necessary. |
| 5. Turn on the machine, the power indicator is on, the fan works, press the welding torch switch and there is gas out, the malfunction LED is on. | - Over-current protection occurs when welding is carried out. - Over-heating protection occurs when welding is carried out. - Some parts on top PCB, center PCB, or bottom PCB are damaged. | - Turn off the machine for 5mins and restart. - Stop the welding operation for 5mins. - Check. Turn off the machine, pull out the connecting cable matching the socket CON7 on top PCB PM-01-A3 with the socket CON3 on bottom PCB PZ-23-A0, turn on the machine. If the malfunction LED is off, the transformer T1 is short-circuited or damaged. If it’s on, turn off the machine, pull out the connecting cable matching socket CON8 on top PCB PM-01-A3, turn on the machine. If the malfunction LED is on, MOSFET VT3.123/VT4./VT5./ VT6. or some parts on drive module PK-07-A1 are damaged; if it’s off, transformer T1/T2/T3 or kenotron VT1—VT12 on center PCB PD-07-B0 is damaged. |
| 6. Turn on the machine and it appears normal, it can start arc, but the weld point appears black. | - The magnet valve or the gas tube is blocked. - The magnet valve is damaged. - Some parts in magnet valve control circuit on bottom PCB PZ-23-A0 are damaged. - The welding torch is damaged. - The tungsten is of bad quality or the argon is impure. | - Clear. - Replace. - Check with a multi-meter if the relay RELY2, MOSFET VT3, resistor R12, diode D8 on bottom PCB is damaged or if the magnet valve control cable matching the socket CON4 is disconnected. - Remove the welding torch and the gas-electricity tie-in, and press the welding torch switch. If there is gas out, the welding torch is damaged. Replace it. - Replace them if necessary. |
| 7. The welding current is unstable and out of control. | - The panel current potentiometer matching the socket CON5 on top PCB PM-01-A3 is in loose connection or is damaged. - The capacitor C5/C6/C11/C12 on bottom PCB PZ-23-A0 leaks or is damaged. - The input cable or output cable is too slim and too long. - Loose connection exists inside the machine. | - Check and replace it if necessary. - Check and replace it if necessary. - Enlarge the cross section area of the cable. - Check. |
| 8. Turn on the machine but it strips. | - The rectifying bridge matching the socket CON5/CON8 on bottom PCB PZ-23-A0 is damaged. - The power supply cable is disconnected or short-circuited. | - Replace. - Check. |
| 9. Press the welding torch switch, the gas valve shuts off at once, and the output voltage shuts off slowly. | - The diode D8 on bottom PZ-23-A0 is damaged. | - Check and replace it if necessary. |
| 10. Press the welding torch switch, there is HF discharge buzz, but no welding voltage output. | - The earth cable of welding torch is in loose connection. - The output terminal of the earth cable is in loose connection with gas-electricity tie-in or with center PCB PD-07-B0. | - Check and replace it if necessary. - Check and replace it if necessary. |
| 11. The arc starting is bad. | - The space between discharge nozzles P1 and P2 on bottom PCB PZ-23-A0 is too big or small, or their surface is badly oxidized. - The high-voltage capacitor C14/C15 on bottom PCB is damaged, or the capacitance becomes smaller. - The tungsten is of bad quality or argon is impure. - The welding torch is loose or broken. - Incorrect turn rate or turn-to-turn electricity leakage problem exists in arc-starting coil matching CP1/CP2. | - Adjust the space between them, or clear their surface. - Check and replace it if necessary. - Check and replace it if necessary. - Check. - Check. |
| 12. Turn on the machine, and it appears normal, but the malfunction LED turns on once welding is carried out. | - The connecting cable matching socket CON6 on top PCB PM-01-A3 with socket CON1 ON center PCB PD-07-B0 is in loose connection. - Loose contact exists in MOSFET VT3./VT4./VT5./VT6. on top PCB PM-01-A3 or kenotron VT1—VT12 on center PCB PD-07-B0. - Some parts on control module PK-02-A1 at position P1 on top PCB are damaged. | - Check. - Check with a multi-meter. - Check if the chip U2 or audion Q9 on PK-02-A1 is damaged. |
| 13. Turn on the machine, and there is HF. | - Some parts in manual switch control circuit on bottom PCB are damaged. - The connecting cable matching socket CON9 on bottom PCB with socket CON6 on top PCB is in loose connection. - The socket CON9 on top CPB is not short-circuited or the ARC/TIG conversion switch of TIG180A is damaged. | - Check with a multi-meter if audion Q1/Q2 on bottom PCB PZ-23-A0 is damaged. Disconnect the connecting cable matching socket CON10, and check if the manual switch board PH-10-A1 is short-circuited. - Check. - Check. |
| 14. Incessant HF exists when welding is carried out. | - The relay RELY1 on bottom PCB PZ-23-A0 is damaged. - Output voltage is interfered by peak wave. | - Check and replace it if necessary. - Screw the feedback cable matching socket CON6 on top PCB with socket CON1 on center PCB. |