MIG series Manual

Marks：Standard by MIG-400

TABLE OF CONTENTS

1：MACHINE STRUCTURE-------------------------------------------------3

2：CIRCUIT DIAGRAM AND PARTS-------------------------------------5

3：FAULT SERVICE-----------------------------------------------------------8

4：POWER-OFF MEASURING-------------------------------------------17

5：POWER-ON MEASURING---------------------------------------------24

6:  WIRE FEEDER MEASURING----------------------------------------78

7：MIG TORCH----------------------------------------------------------------34

8：REPAIR AND MAINTENANCE ---------------------------------------36

1.1 Front pane

1.2 Back panel

1.3 Top side

1.4 Right side

1.5 Left side

1.6 Repair tools

2.1a CIRCUIT DIAGRAM

2.1b FRONT PANEL CIRCUIT DIAGRAM

2.2 COMPONENT LIST OF CIRCUIT DIAGRAM

| Position | Description | Model |
| --- | --- | --- |
| C4,C5 | Polypropylene capacitors | 20UF/1400V DC |
| C6,C7 | Polypropylene capacitors | 50UF/450V AC |
| C8 | Polypropylene capacitors | 4UF/500V AC |
| D1 | 3phs rectifier bridge | MDS100-16 |
| D2,D3 | Fast recovery diodes | DH2F100N4S |
| FU1 | Power fuse | 5A |
| FU2 | Heaters | T60 375 |
| IFB | Feedback board | MIG-350 |
| XZ | Absorb capacitor board | MIG-350 |
| L1 | Inductance | MIG-350 |
| L2 | Filter inductance | MIG-350 |
| L3 | Syntony inductance | MIG-350 |
| L4 | Inversion inductance | MIG-350 |
| L5 | Out put reactor | MIG-350 |
| F | Axial flow fan | TG22580HA2(AC220V) |
| G1,G2,G3,G4 | IGBT | GD75HFL120C1S |
| +,- | Out put terminal | DKM35-50 |
| QD | Driver board | MIG-350 |
| KZ1 | Control board | MIG-350 |
| KZ2 | Control board | ARC400 |
| T1 | Main transformer | MIG-350 |
| T3 | Power transformer | T3 |
| T4 | Power transforme | T4 |
| T5 | Power transforme | T5 |
| SW1,SW2 | Temperature switch | Closed 70° |
| FL | Shunt | 300A/75Mv |
| J1,J2 | Relay | HH52P(AC24V) |
| K1 | Air switch | DZ47 D40A |

2.3 Failure by the damage component

| Damage component | Failure phenomenon |
| --- | --- |
| FU1 | Digital meter not working，machine no out put |
| FU2 | CO2 heaters is not heating |
| Contac tip | Welding current instability，big spatter，poor of molding，instability wire feed，Wire sticking contact tip |
| Wire feed hose | Welding current instability，big spatter，poor of molding，instability wire feed |
| Wire feeder wheel | Welding current instability，big spatter，poor of molding，instability wire feed, no wire feed |
| Wire feed tractor | No wire feed，Intermittent wire feed |
| Torch switch | Machine not working，press switch wire feed too speed，boot-strap wire feed |
| Control cable（6pins） | Burn main control board, Current and voltage can not be adjusted，no wire feed，gas valve not working，press torch witch the machine not worling |
| Voltage feedback line off | MIG state，Max out put voltage，can not adjustted |
| Current feedback line off | MIG state，slow wire feed，current instability |
|  | MMA state，Max out put voltage，can not adjustted |

3 Fault repair

3.1 Warning

Welding machine should be repaid by professional, any error connector and fixing will cause PCB or components damage ，after cut of power ，need to wait a few minutes ，after confirmed input capacitance fully discharge，should can starting repair，pro-tected from electric shock。

3.2 Notes

⑴ Please be Sure to observe the manual notes ,otherwise might occur accident.

⑵ Repair the machine by the professional qualifications staff.

⑶ Repair ,please refer to following tips and maintenance of the fault phenomena, if  can not repair, please contact the manufacturer to avoid the spreading of the accident.

⑷ Be fore repair , please make sure the mahine’s power supply is normal.

Did the power supply voltage and frequency same as the nameplates，the Power supply capacity can meet the demand of the machine working ，avoid the welding cable and control cable, wire breakage; prevent welding output short circuit.

3.3 Content prompt

MIG state

A: After starting machine power light is off, digital meter not show --------------------------------9

B: Digital meter display abnormal -------------------------------------------------------------------------10

C: Starting display normal ,but can not welding ，no voltage current output--------- --------10

D: MIG torch or welding cable too hot--------------------------------------------------------- -----------10

E: No wire feed---------------------------------------------------------------------------------- ---------------10

F: Tractor rotation，but can not wire feed ----------------------------------------------------------------11

G: Instability wire feed -----------------------------------------------------------------------------------------11

H: After energized ,no press torch switch, automatic wire feed--------------------------------------11

I: Speed wire feed in welding process, can not welding -------------------------- --------------------11

J: In actually welding process, welding current instability --------------------------------------------12

K: Welding current、Mix voltage，can not adjustment ，no fuse ---------------------------------12

L: Welding current 、Max voltage，can not adjustment ---------------------------------------------12

M: Welding produce bubbles, gas hole -------------------------------------------------------------- ----13

N: Shallow melting depth rate, weld-seam too high ---------------------------------------------------13

O: Big splash, welding wire burst, hard of forming ----------------------------------- -----------------13

P: Strike fire between the nozzle and the workpiece --------------------------------------------------14

Q: Gas impassable or gas small flow  -------------------------------------------------------------------14

R: Long ventilation ----------------------------------------------------------------------------------------------14

S: CO2 Gas regulator without heating ---------------------------------------------------------------------14

T: Overheat lights on，can not welding --------------------------------------------------------------------15

U: After electricity，air switch（K1）trip--------------------------------------------------------------------15

MMA/TIG states

A: After starting machine power light is off, digital meter no display ------------------------15

B: Digital meter display abnormal --------------------------------------------------------------------15

C: Starting machine normal ,but can not weldin，no voltage output ------------------------16

D: Starting machine normal , False output voltage，welding rod contact with the workpiece，have          small spark, but can not welding------------------------------------------------------------------------16

E: Big splash in welding, bad forming ---------------------------------------------------------------16

F: Max welding current，can not adjustment ------------------------------------------------------16

G: Min welding current ，can not adjustment ------------------------------------------------------17

3.4 Repair ways:

MIG states    A: After starting machine power light is off, digital meter not show

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Power source missing phs ？ | Check the 3phs input power |
| 2 | Air switch broken（K1）? | Check the 3phs input power |
| 3 | Fuse is blown（FU2）? | Inside the machine, check and replace |
| 4 | T4 Power transformer broken？ | check and replace（refer to 4.1A，5.1A） |
| 5 | Power switch broken？ | Check and replace |
| 6 | Digital meter broken or connect line loose? | Check connect line and replace |
| 7 | Main control board（KZ1）broken？ | Check and replace |

MIG states    B: Digital meter display abnormal

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | J1 relay contact not good？ | Check and replace |
| 2 | Feeder cable connection not good or short-term？ | Recovery or replace（refer to 6.2） |
| 3 | Remote control box broken？ | Check and replace（refer to 6.2，6.3） |
| 4 | Digital meter broken？ | Replace digital meter |
| 5 | Main control board （KZ1）broken？ | Replace |

MIG states   C: Starting machine normal, but can not welding ，no voltage current out put

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Welding cable break or not connect good？ | Reliable connection |
| 2 | 3phs rectifier bridge short circuit？ | Check and replace |
| 3 | IGBT broken？ | Check and replace |
| 4 | Fast recovery diode（D2,D3）broken? | Check and replace |
| 5 | Absorbing capacitor board（XZ）broken？ | Check and replace |
| 6 | Main control board（KZ1）,driver board（QD） broken？ | Check and replace |

MIG States   D:MIG torch or welding cable too hot

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Welding current too large？ | Change welding torch |
| 2 | Welding cable too small or not connect good？ | Tight connect or change welding cable |
| 3 | Loose contact tip？ | Tight contact tip |

MIG State   E: No wire feed

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Torch switch damage、short circuit？ | Repair or replace |
| 2 | Wire feeder control cable connector not good？ | Check control cable line1#，6# |
| 3 | Wire feeder tractor damage？ | Check and replace |
| 4 | Power transformer T4 damage？ | Check and replace |
| 5 | Main control board (KZ1)damage? | Check and replace |

MIG state    F:Tractor rotation，can not wire feed

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Wire fuse with the contact tip？ | Change contact tip |
| 2 | The wheel’s slot not suitable for the wire？ | Change wire feeder wheel |
| 3 | The wheel’s pressure not enough，wheel slipping？ | Adjust wire feeder wheel’s pressure |
| 4 | Wire feed’s tube block？ | Clean or replace the wire feed tube |
| 5 | The Wire bending deformation？ | Cutting the wire，install wire again |

MIG state   G: Wire feed instability

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | The current and voltage not match？ | Adjust current and voltage |
| 2 | The quality of wire is not good？ | Change the wire |
| 3 | Contact tip serious wear and tear？ | Change contact tip |
| 4 | Wire feeding tube’s quality problem not matching with the diameter | Change wire feed’s tube |
| 5 | Welding torch bending diameter＜300mm ? | Straightened troch |
| 6 | pressure wire wheels too loose or too tight？ | Adjust the wheel’s pressure |
| 7 | The wheel wear and tear？ | Change the wheel |
| 8 | Wire feeder’s control connect not good | Checking and resume |
| 9 | Wire feeder’s tractor damage？ | Check and replace |
| 10 | Main control board（KZ1）damage？ | Check and replace |

MIG state    H: After energized ,no press torch switch, automatic wire feed

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | torch switch line short-circuited？ | Check and repair |
| 2 | Control plug socket short-circuited？ | Check and repair |
| 3 | Main control board（KZ1）broken？ | Check and repair |

MIG state        I: during welding wire feed too fast, can not welding

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Torch switch connect not good？ | Check and replace |
| 2 | Wire feeder control cable short circuit ？ | Check and repair |
| 3 | Control board（KZ1）damage？ | Check and replace |

MIG state       J: Actually during welding ，Welding current instability

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Welding current voltage adjust not matching？ | Re-adjust the parameter |
| 2 | Missing phase or the voltage of 3phs instability？ | Checking power source |
| 3 | Out put cable connect not good？ | Checking and resume |
| 4 | Wire feed instability？ | Refer to 3.3G |
| 5 | Control box’s potentiometer broken？ | Check and replace |
| 6 | The shunt’s （FL） feedback line connect not good.？ | Check and repair |
| 7 | Main control board（KZ1）broken？ | replace |

MIG State     K: welding current、mix voltage，can not adjust，no fuse

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Welding cable break off？ | Check and repair |
| 2 | Control box’s potentiometer damage ？ | Check and replace |
| 3 | Inductance potentiometer（L）broken？ | Check and replace |
| 4 | Machine inside voltage, current feedback off line？ | Check and repair |
| 5 | IGBT broken？ | Check and replace |
| 6 | Fast recovery diode（D2、D3） broken？ | Check and replace |
| 7 | Main circuit short circuit or not connect good？ | Check and repair |
| 8 | Line’s of feedback board（IBF） not connect good？ | Check and repair |
| 9 | Main control board（KZ1）or driver board（QD） broken？ | Check and replace |

MIG state   L:welding current、Max voltage，can not adjustment

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Control box’s potentiometer damage？ | Check and replace |
| 2 | Machine inside voltage, current feedback off line？ | Check and repair |
| 3 | Control board（KZ1）broken？ | Check and replace |

MIG state   M: Welding produce bubbles, gas hole

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Welding voltage too high？ | Adjust the appropriate voltage |
| 2 | Workpiece or welding wire rust, oil？ | Clear the workpiece and wire |
| 3 | Spatters sticky one the nozzle, gas can not flowing？ | Clear the spatters |
| 4 | Wire extend too long？ | Keep a suitable length during welding |
| 5 | Gas flow is inappropriate？ | Adjusting the gas flow and keep clean |
| 6 | The gas impure , have water？ | Change the gas |
| 7 | Gas calve broken？ | Replace gas valve |
| 8 | Main control board（KZ1）broken？ | Check and replace |

MIG state    N: Shallow melting depth rate, weld-seam too high

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Welding current voltage not matching？ | Adjust appropriate current voltage welding |
| 2 | Wire extend not inappropriate？ | Keep a suitable length during welding |
| 3 | Contact tip too short？ | Change contact tip |
| 4 | Inductance（L）adjust inappropriate？ | Adjust the appropriate parameters. |
| 5 | Mai control board（KZ1）broken？ | Check and change |

MIG state   O: Big splash, welding wire burst, hard of forming

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Welding voltage and current specifications inappropriate？ | Adjust appropriate current voltage welding |
| 2 | The inductance（L）adjustment too small？ | Increasing inductance |
| 3 | Connect reversed polarity？ | Change the polarity |
| 4 | CO2 gas impurity？ | Change gas |
| 5 | Workpiece or welding wire rust, oil？ | Clear the workpiece and wire |
| 6 | Torch angle is too large？ | Adjust torch angle |
| 7 | Wire extend too long？ | When welding adjust the distance |
| 8 | Contact tip serious wear and tear？ | Change contact tip |
| 9 | Machine inside voltage, current feedback connect not good？？ | Reliable connection |
| 10 | Main control board（KZ1）fault？ | Check and change |

MIG state       P: Strike fire between the nozzle and the workpiece

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Spatter accumulation between the nozzle? | Clean Spatter |
| 2 | Conductive rod and nozzles short circuit？ | Check the circuit phenomenon |
| 3 | Nozzle insulation burn? | Check and change |

MIG state    Q: gas block or gas flow is small

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Severity air leakage？ | Change gas tube |
| 2 | gas bottle pressure too low？ | Change the gas bottle |
| 3 | The spatter obstruction gas nozzle？ | Cleaning nozzle’s spatter |
| 4 | Reducing valve to freeze or broken？ | Connected preheat power or replace Reducing valve |
| 5 | Solenoid valve's air inlet was blockage.？ | Removing foreign |
| 6 | The valve’s spring is block unable to close？ | Open valve change the spring |
| 7 | Solenoid valve is shorted or damaged | Change the gas valve |
| 8 | Main control board（KZ1）broken？ | Check and change |

MIG state    R: Long ventilation

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Move panel switch to the gas inspect position ? | Adjust switch’s position |
| 2 | The valve jacking up by the foreign matter ？ | Open the valve checking and repair |
| 3 | Main control board（KZ1）broken？ | Check and change it |

MIG state    S:CO2 Gas regulator not heating

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Gas regulator broken？ | Change and repair |
| 2 | Regulator’s power socket damage？ | Replace |
| 3 | Fuse FU2 broken ？ | Replace |
| 4 | Regulator’s power socket connect line off.？ | Check and repair |
| 5 | Transformer T5 broken？ | Check and replace |

MIG state   T: Overheating light on, can not welding

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Big current welding in a long time？ | Reduce the welding current |
| 2 | Fan broken？ | Check and replace |
| 3 | Temperature switch broken？ | Check and replace |

MIG state     U: After power up, the air switch (K1) trip

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | 3phs rectifier bridge（D1）short circuit？ | Check and replace |
| 2 | Filter capacitor C4,C5 broken？ | Check and replace |
| 3 | IGBT broken？ | Check and replace |

MMA/TIG state A: after starting machine light of power source off, digital meter no display

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Power source missing phase？ | Checking 3phs power source |
| 2 | Air switch（K1）broken? | Check each phase of the air switch is good or not |
| 3 | Fuse（FU2） break off? | Inside the machine，check and replace |
| 4 | Power transformerT4 broken？ | Check and replace（refer to 4.1A，5.1A） |
| 5 | Digital meter damage or connect line loose？ | Check connect line or replace |
| 6 | Main control board（KZ2） broken？ | Check and replace |

MMA/TIG state    B: Digital meter display abnormal

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | J1 potentiometer not connect good？ | Check and replace |
| 2 | Digital meter broken？ | Replace digital meter |
| 3 | Main control board（KZ1）broken？ | replace |

MMA/TIG state   C: starting display normal, but can not welding ，no voltage out put

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | 3phs power source missing phase？ | Check and repair |
| 2 | Welding cable break off or not connect good？ | Reliably connected |
| 3 | 3phs rectifier bridge circuit？ | Check and replace |
| 4 | IGBT broken？ | Check and replace |
| 5 | Fast recovery diode（D2,D3） broken? | Check and replace |
| 6 | Absorbing capacitor board（XZ） broken？ | Check and replace |
| 7 | Main control board（KZ2）,driver board （QD）broken？ | Check and replace |

MMA/TIG state D: starting display normal, False voltage output，welding rod contact with the workpiece ，small sparklet, but can not welding

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | IGBT broken？ | Check and replace |
| 2 | Fast recovery diode（D2,D3）broken? | Check and replace |
| 3 | Main control board（KZ2）,driver board（QD） broken？ | Check and replace |
| 4 | Current adjust potentiometer broken？ | Check and replace |

MMA/TIG state		E: Big splash during welding, bad molding

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | 3phs missing phase ？ | Check and repair |
| 2 | Output polarity reversal？ | Change the polarity |
| 3 | Secondary output line connect not good？ | Tight connector |

MMA/TIG state  F:Max welding current, can not adjustment

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Inside machine current feedback line off？ | Check and repair |
| 2 | Control board（KZ2）broken？ | Check an replace |

MMA/TIG State G: Mix current，can not adjustment

| No | Fault cause | Differentiate ways and processing |
| --- | --- | --- |
| 1 | Potentiometer of current adjustment is broken？ | Check and replace |
| 2 | Control board（KZ2） broken？ | Check and replace |

4 Power off measuring

Warning：

(1) Make sure 3phs power cable are completely out of power, should begin detected the components

⑵Welding cable is disconnect from the output terminal

⑶ After a few minute，check input capacitance，3phs rectifier，IGBT components，make sure full discharges should can begin measuring, to prevent electrical shock.

Note matters：

1.  Digital Multimeter state diode / resistor / capacitor gear position

2.  Using different multimeter for measuring, the value are different.

3.  After the digital stable on the multimeter, should can recorded.

4.  Make sure the myltimeter’s pen and measurement point in good connector

5.  Measurement parts of components need cut off with the control board connector

4.1Conten Prompt：

A: Power transformer T4 measuring

B: Power transformeT5 measuring

C: Power transforme T6 measuring

D: Diverter FL measuring

E: Temperature switch measuring

F: Inductance adjusting potentiometer measuring

G: Closing arc current potentiometer measuring

H: Closing arc voltage potentiometer measuring

I: MMA current potentiometer measuring

J: State switch measuring

K: Torch(standard）switch measuring

4.1 A: power transformerT4 measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | A1(1) | A1(2) | 2-4Ω | A1(1)—T4(6) A1(2)—T4(5) A1(3)—T4(7) |
|  | A1(1) | A1(3) | 2-4Ω |  |
|  | A1(2) | A1(3) | 4-8Ω |  |
|  | A10(1) | A10(3) | 2-4Ω | A10(1)—T4(8) A10(2)—T4(10) A10(3)—T4(9) |
|  | A10(2) | A10(3) | 2-4Ω |  |
|  | A10(1) | A10(2) | 4-8Ω |  |
|  | T4(3) | T4(4) | 2.5-4Ω | Meter supply source |

Notes：before measuring, take off MIG control board（KZ1）A1 and A10plug

B: Power transformer T5 measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | A2(3) | A2(4) | 2-4Ω | A2(3)—T5(9) A2(4)—T5(10) |
|  | A2(1) | A2(5) | 0.2-0.5Ω | A2(1)—T5(7) A2(5)—T5(6) A2(7)—T5(8) |
|  | A2(1) | A2(7) | 0.2-0.5Ω |  |
|  | T5(4) | T5(5) | 0.2-0.5Ω | Heating power source |

Notes：Before measuring，please take off the plug of MIG main control board （KZ1）

C: Power transformer T6 measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | A6(5) | A6(6) | 4-6Ω | A6(5)—T6(5) A6(6)—T6(6) |
|  | A2(2) | A2(3) | 2-3Ω | A2(2)—T6(13) A2(3)—T6(14) |
|  | A2(4) | A2(6) | 2-3Ω | A2(4)—T6(10) A2(5)—T6(12) A2(6)—T6(11) |
|  | A2(5) | A2(6) | 2-3Ω |  |

Notes：Before measuring，please take off the plug of MMA main control board （KZ2）A2,A6

D: Diverter FL measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | FL+ | FL- | ＜0.5Ω | A7(4)—FL+ |
|  | A7(4) | A7(3) |  | A7(3)—FL- |

Notes：Before measuring, please take off the plug of MIG main control board（KZ1）A7

E: Temperature switch measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | A5(1) | A5(2) | ＜1 Ω | MIG control board（KZ1） |
|  | A4(1) | A4(2) |  | MMA control board（KZ2） |

Notes：Before measuring，please take off the plug of main control boar（KZ1）A5,and MMA main control board（KZ2）A4

F: Inductance adjusting potentiometer measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | A8(1) | A8(2) | 5Ω—33K | Adjusting L potentiometer，Resistance changes continuously |

Notes：Before measuring，please take off the plug of MIG main control（KZ1）A8

G: Collect arc current potentiometer measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | A6(2) | A6(3) | 5Ω—4.7K | Adjustment potentiometer，Value continue to change |
|  | A6(2) | A6(5) | 5Ω—4.7K |  |

Notes：Before measuring，please take off MIG main control board （KZ1）A6

H: Collect voltage potentiometer measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | A6(1) | A6(4) | 5Ω—4.7K | Adjustment potentiometer，Value continue to change |
|  | A6(1) | A6(5) | 5Ω—4.7K |  |

Notes：Before measuring，please take off the plug of MIG main control board （KZ1）A6

I:MMA current potentiometer measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | A1(2) | A1(5) | 5Ω—4.7K | Adjustment potentiometer，Value continue to change |
|  | A1(2) | A1(6) | 5Ω—4.7K |  |

Notes：before measuring，please take off the plug of MMA main control board （KZ2）A1

J: State switch measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | A4(3) | A4(4) | 0Ω |  |
|  | A4(3) | A4(4) | 0.L |  |
|  | A4(6) | A4(3) | 0.L |  |
|  | A4(6) | A4(3) | 0Ω |  |
|  |  |  | J1，J2 potentiometer close |  |
|  |  |  | J1，J2potentiometer open |  |

Notes： Before measuring，please take off the plug of MIG main board（KZ1）A4

K: Torch（standard）switch measuring

| Multimeter range | Red pen-shape meter | Black pen-shape meter | Reference value | remark |
| --- | --- | --- | --- | --- |
| Resistance stalls auto range | GS1(1) | GS1(2) | open（＜1Ω） | Press switch |
|  | GS1(1) | GS1(2) | close（O.L） | Release switch |

5 Power on measuring

WARNING：

This measuring only in the air switch not tripped，if air switch has been tripped, it should be detected in the power-off state.

Electric shocks can be fatal：

⑴The inside of the machine high voltage up to 600V, do not touch live parts.

⑵Ensure that physical and work pieces, ground insulation between each other.

⑶Please use a dry, non-destructive, such as insulated gloves to protect.                     Moving parts can cause injury to human：

⑴Rotating parts (such as fans) will hurt your fingers or loose clothing caught.

⑵When aerial work, use safety nets

Notice：

⑴Digital multimeter state in " AC/DC voltage ".

⑵Measurements on the use of different multimeter,numerical difference.

⑶Digital multimeter stable for record.

⑷Ensure that the multimeter pen-shape metre and detection points connected.

⑸In this manual ,test data is in the power supply voltage of AC380V, if in situ measurements of voltages different from the manual, make sure the supply voltage.

5.1 A:Transformer T4 ---voltage measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| AC voltage Automatic text range | A1(1) | A1(2) | 19V±10% | A1(1)—T4(6) A1(2)—T4(5) A1(3)—T4(7) |
|  | A1(1) | A1(3) | 19V±10% |  |
|  | A10(1) | A10(3) | 19V±10% | A10(1)—T4(8) A10(2)—T4(10) A10(3)—T4(9) |
|  | A10(2) | A10(3) | 19V±10% |  |
|  | T4(3) | T4(4) | 10V±10% | Metet power supply |

B:Transformer T5---voltage measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| AC voltage Automatic text range | A2(3) | A2(4) | 26V±10% | A2(3)—T5(9) A2(4)—T5(10) |
|  | A2(1) | A2(5) | 26V±10% | A2(1)—T5(7) A2(5)—T5(6) A2(7)—T5(8) |
|  | A2(1) | A2(7) | 26V±10% |  |
|  | T5(4) | T5(5) | 36V±10% | Heating power |

C:Transformer T6 ---voltage measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| AC voltage Automatic text range | A6(5) | A6(6) | 38V±10% | A6(5)—T6(5) A6(6)—T6(6) |
|  | A2(2) | A2(3) | 19V±10% | A2(2)—T6(13) A2(3)—T6(14) |
|  | A2(4) | A2(6) | 19±10% | A2(4)—T6(10) A2(5)—T6(12) A2(6)—T6(11) |
|  | A2(5) | A2(6) | 19±10% |  |

D:No-load voltage---measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| DC voltage Automatic text range | OUTPUT + | OUTPUT_ | 65V±10% | MIG take on torchs witch |
|  | OUTPUT + | OUTPUT_ | 70V±10% | MMA/TIG state |

6 Wire feeder measuring

6.1A Single-driven wire feeder

6.1B Euro double-driven wire feeder

6.1C Remote control  wiring diagram

6.2 Power off measuring

warning：

⑴After the three-phase power cable to completely disconnect, you can begin testing components

⑵The welding cable disconnected from the output terminals

⑶Wait a few minutes, measurement input capacitors, three-phase bridge ,IGBT and so on, make sure completely discharge, to prevent electric shock.

Notice：

⑴Digital multimeter state in " Diode/Resistance ".

⑵Measurements on the use of different multimeter,numerical difference.

⑶Digital multimeter stable for record.

⑷Ensure that the multimeter pen-shape metre and detection points connected.

(5)Measuring need be disconnected with welding machine, direct measure control cable plug.

6.2.1  Wire feed motor DJ voltage—measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| Resistance stalls | KC(6) | KC(1) | 4.7—5.5 |  |

6.2.2 Solenoid valve DF---measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| Resistance stalls | KC(6) | KC(2) | 50—90 |  |

6.2.3 Torch switch、manual wire switch ---maesuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| Resistance stalls Automatic range | KC(5) | KC(3) | open（＜1Ω） | Take on torch switch |
|  | KC(5) | KC(3) | close（O.L） | Take off torch switch |
|  | KC(3) | KC(5) | 9-11K | Take on manual wire switch |
| Diode stalls | KC(5) | KC(3) | 0.8V±10% | Take off manual wire switch |

    - Given voltagle potentiometer RV---measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| Diode stalls | KC(5) | KC(4) | 0.55V—O.L | Potentiometers to djust from small to the largest |

Notice：Adjust the potentiometer,the resistance value should continuous change, no jumping

6.2.5 Given current potentiometer RI---measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| Diode stalls | KC(4) | KC(5) | 0.55V—O.L | Potentiometers to adjust from small to the largest |

Notice：Adjust the potentiometer,the resistance value should continuous change, no jumping

6.3 Power on measuring

WARNING：

This measuring only in the air switch not tripped，if air switch has been tripped, it should be detected in the power-off state.

Electric shocks can be fatal：

⑴The inside of the machine high voltage up to 600V, do not touch live parts.

⑵Ensure that physical and work pieces, ground insulation between each other.

⑶Please use a dry, non-destructive, such as insulated gloves to protect.

Moving parts can cause injury to human：

⑴Rotating parts (such as fans) will hurt your fingers or loose clothing caught.

⑵When aerial work, use safety nets

Notice：

⑴Digital multimeter state in " AC/DC voltage ".

⑵Measurements on the use of different multimeter,numerical difference.

⑶Digital multimeter stable for record.

⑷Ensure that the multimeter pen-shape metre and detection points connected.

⑸In this manual ,test data is in the power supply voltage of AC380V, if in situ meas

urements of voltages different from the manual, make sure the supply voltage.

6.3.1 Switch singal—measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| AC voltage Automatic text range | A3(6) | A3(8) | 0V | Take on switch for torch |
|  | A3(6) | A3(8) | 9V±10% | Press the manual wire feed switch |
|  | A3(6) | A3(8) | 14V±10% | Normal not operation |

Notice：MIG control board（KZ1）

6.3.2 Solenoid valve DF—measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| AC voltage Automatic text range | A3(1) | A3(3) | 24V±10% | Take on switch for torch |
|  | A3(1) | A3(3) | 0V | Take off switch for torch |

Notice：MIG control board（KZ1）

6.3.3 Wire feed motor DJ voltage—measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| AC voltage Automatic text range | A3(2/3) | A3(4/5) | 1-24V±10% | Welding condition（Adjustable current） |
|  | A3(2/3) | A3(4/5) | 28V±10% | Manual wire feeding condition |
|  | A3(2/3) | A3(4/5) | 1-8V±10% | Slow wire feed（Adjustable current） |

Notice：MIG control board（KZ1）

6.3.4 Given signal—measuring

| Multimeter measuring range | Red pen-shape metre | Black pen-shape metre | reference value | Remark |
| --- | --- | --- | --- | --- |
| AC voltage Automatic text range | A3(7) | A3(8) | 7VAC±10% | Max current ,Max voltage |
|  | A3(7) | A3(8) | 4.2VAC±10% | Min current ,Max voltage |
|  | A3(7) | A3(8) | 0.7VAC±10% | Min current ,Min voltage |

Notice：MIG control board（KZ1）

7 MIG torch

7.1 Panasonic MIG torch

7.2 Euro MIG torch

7.3 Wire feed tube

8 Repair and maintenance

WARNING：

1：Welding machines require professional maintenance

2：The inside of the machine high voltage up to 600V, do not touch live parts; Detect the components in machine,it should completely turn off the power. After take off the power, wait a few minutes, measuring the voltagle  on DC bus capacitor ,make sure replacement the components after it completely discharge.

3：IGBT is sensitive to static electricity, static electricity  will damage module during the operation process。

4：High power module, screw should be uniform.

8.1 Repair and maintenance

1：Every 3-6 months, professionals people use compressed air to dust removal of welding power source, as well as check the fasteners don’tloosein machine. Every 1 month in bad environment.

2：Three-phase power supply voltages are within 340V-420V, make sure missing phase.

3：Make sure reliable earthing and insulation.

4：Often check cable breakage, disconnection, adjustment knob is loose.

5：Check the cable connection to the welding machine terminals for fastening and reliable, avoid damage to the terminals.

6：Check the contact tip, pressure wire wheel and wire wheel,if it be damaged,it should be replaced, often cleaning wire feed hose, ensure the wire feed flow.

7：Check the welding torch not breakage, disconnection, check gun head, nozzle,  gas sieve device is loose, burning.

8：Often check the pipe for damage, whether gas table with frosting, not heating.

9：Keep the welding machine with good ventilation, internal welding machine drying.