WP-17V-4(10-25)

1. 产品描述：

| Torch Specification |  | Standard GB/T15579.7-2005 |  |
| --- | --- | --- | --- |
| Rated current | 140A/DC,125A/AC | Rated duty cycle | 35% |
| Protection gas | Ar | Arc starting voltage | 3000V |
| Gas flow | 4-15l/min | Welding type | TIG welding |
| Welding methods | Manual | Torch back connector | 25mm2 fasten connector |
| Rated volage | ≤113V | Length | 4m |
| Stabilizing arc voltage | ≤113V |  |  |

2. 整枪结构图及清单：

| Item | Code | Description | Qty(pc) | Remarks |
| --- | --- | --- | --- | --- |
| 1 | IGZ0662 | Torch head TIG17/with gas valve | 1 |  |
| 2 | IHJ0011 | Long torch cap TIG17-18-26 | 1 |  |
| 3 | IGV0009 | Straight torch handle TIG9-17 | 1 |  |
| 4 | IGN0005 | Gas and electric hose 4M M12x1 TIG9/17/24 | 1 |  |
| 5 | ISQ6675 | TIG fasten connector 25 M12x1 with 2m length | 1 |  |

3. Parts list：

| Item | Code | Description | Qty(pc) | Remarks |
| --- | --- | --- | --- | --- |
| 1 | IHJ0012 | Short torch cap TIG17-18-26 | 1 |  |
| 2 | IGF0006-24 | Electrode holder Φ2.4 TIG17-18-26 | 1 |  |
| 3 | IGU0006-24 | Electrode holderΦ2.4 50mm TIG17-18-26 | 1 |  |
| 4 | IGU0006-16 | Electrode holderΦ1.6 50mm TIG17-18-26 | 1 |  |
| 5 | IGS0007-02 | Ceramic nozzle/GR4 /TIG17/18/26 | 1 |  |
| 6 | IGS0008-02 | Ceramic nozzle/GR5 /TIG17/18/26 | 1 |  |
| 7 | IGS0009-02 | Ceramic nozzle/GR6/TIG17/18/26 | 1 |  |
| 8 | IRH0073 | Plastic box | 1 |  |

4．检测设备：

| 序号 | 名称 | 型号 | 编号 | 校准有效期 | 正在使用 |
| --- | --- | --- | --- | --- | --- |
| 1 | 电气回路测试台 | 自制 | IN-SB-04 | 2011.12.1-2012.12.1 | √ |
| 2 | 耐压试验仪 | GY2676 | IN-SB-05 | 2011.12.1-2012.12.1 | √ |
| 3 | 气体流量计 | LZB-25 | IN-SB-07 | 2012.4.1-2013.4.1 | √ |

5. 检验标准：

| 试验检测要求 |  |
| --- | --- |
| 检验项目 | 标准要求（GB/T15579.7-2005弧焊设备安全要求 第7部分：焊炬） |
| 标记 | 焊炬上应清晰并永久地标出以下信息： |
|  | a. 制造厂给定的型号 |
|  | b. 手柄标准号，以确定枪柄符合本标准要求 |
| 绝缘电阻 | 焊接回路对焊炬的绝缘体外表面≥1MΩ； |
|  | 控制回路对焊炬的绝缘体外表面≥1MΩ； |
|  | 控制回路对焊接回路≥2.5MΩ； |
| 介电强度 | 控制回路对焊炬绝缘体外表面应能承受1000V的高频交流电压,历时60s,无闪络或击穿现象 |
|  | 焊接回路对焊炬的绝缘体表面应能承受3000V的高频交流电压,历时2S,无闪络或击穿现象 |
|  | 控制回路对焊接回路应能承受3000V的高频交流电压,历时2S,无闪络或击穿现象 |
| 温升 | 电缆软管组件的外表面任何一处的温升不应超过40K |
|  | 焊炬（枪）手柄手持位置的外表任何一处温升不超过30K |
|  | 试验后，焊炬（枪）的安全性和操作性能不应降低 |
| 热额定性能 | 焊炬（枪）的负载持续率35% |