WP-26-4(WSME200)

1.Specification：

| Torch specification |  | Standard GB/T15579.7-2005 |  |
| --- | --- | --- | --- |
| Rated current | 180A/DC,130A/AC | Rated duty cycle | 35% |
| Protection gas type | Ar | Arc starting voltage | 3000V |
| Gas flow | 4-15l/min | Welding type | TIG welding |
| Welding methods | Manual | Torch back connector | M16X1.5 |
| Rated voltage | ≤113V | Cable length | 4m |
| Stabilizing arc voltage | ≤113V |  |  |

2. Torch explosive drawing and sparts list：

| Item | Code | Description | Qty(PC) | Remarks |
| --- | --- | --- | --- | --- |
| 1 | IGZ0071 | Torch head TIG26 | 1 |  |
| 2 | IHJ0011 | Long torch cap TIG17-18-26 | 1 |  |
| 3 | IHJ0598 | Cover of swith/TIG handle/four switch | 1 |  |
| 4 | IHJ0986 | Buttons set/Four switch/Orange/WINN | 1 |  |
| 5 | IHQ0177 | PC board/four switch/WTL | 1 |  |
| 6 | IHQ0188 | OMNON small switch for TIG | 4 |  |
| 6.1 | IHQ0185 | 6pins connector/Male/PCB | 1 |  |
| 7 | IGV0680 | Ergao TIG handle grey/rubber | 1 |  |
| 8 | IFT0853 | Screw D.3x8 | 5 |  |
| 9 | IHJ0894 | TIG ball type connector set /big/new type | 1 |  |
| 10 | IGN0002 | Gas and electric hose 4M M12x1 TIG26 | 1 |  |
| 11 | IFT6002 | Label TIG26 | 1 |  |
| 12 | IHQ0181 | 6针 connector/female/PCB | 1 |  |
| 13 | IHQ0182 | pins/6pins connector | 6 |  |
| 14 | IHQ0636 | Control cable 6x0,3mmq PVC AL/m | 4.3m |  |
| 15 | IKH0094 | EPDM cover D.24X26 | 1 |  |
| 16 | IHQ0792 | 14pins connector/AMP | 1 |  |
| 17 | IZT9000 | Connector belt5/8-18UNF screw | 1 |  |
| 18 | IFT0737 | Cable clam dia 9.5 | 2 |  |
| 19 | ISQ6022 | Connector copper set 35-50 for TIG with M8 connector(Longer） | 1 |  |
| 20 | IHJ0856 | Rubber TRAK SK 35-70 | 1 |  |
| 21 | IKH0605 | EPDM cover D.5x8 red | 0.6m |  |
| 22 | IFT0871 | Plastic connector part 90 degrees M8X1.25 | 1 |  |

3. Parts list：

| Item | Code | Description | Qty(pc) | Remarks |
| --- | --- | --- | --- | --- |
| 1 | IGF0006-32 | Collect body Φ3.2 TIG17-18-26 | 1 |  |
| 2 | IGS0007-02 | Ceramic cup GR4 TIG17-18-26 | 1 |  |
| 3 | IGS0008-02 | Ceramic cup GR5 TIG17-18-26 | 1 |  |
| 4 | IGS0009-02 | Ceramic cup GR6 TIG17-18-26 | 1 |  |
| 5 | IGS0005-02 | Ceramic cup GR7 TIG17-18-26 | 1 |  |
| 6 | IGU0006-16 | Collect Φ1.6 50mm TIG17-18-26 | 1 |  |
| 7 | IGU0006-24 | Collect Φ2.4 50mm TIG17-18-26 | 1 |  |
| 8 | IGU0006-32 | Collect Φ3.2 50mm TIG17-18-26 | 1 |  |
| 9 | IHJ0012 | Short torch capTIG17-18-26 | 1 |  |
| 10 | IRH0073 | Plastic box | 1 |  |

4．检测设备：

| 序号 | 名称 | 型号 | 编号 | 校准有效期 | 正在使用 |
| --- | --- | --- | --- | --- | --- |
| 1 | 电气回路测试台 | 自制 | IN-SB-04 | 2011.12.1-2012.12.1 | √ |
| 2 | 耐压试验仪 | GY2676 | IN-SB-05 | 2011.12.1-2012.12.1 | √ |
| 3 | 气体流量计 | LZB-25 | IN-SB-07 | 2011.5.6-2013.5.5 | √ |

5. 检验标准：

| 试验检测要求 |  |
| --- | --- |
| 检验项目 | 标准要求（GB/T15579.7-2005弧焊设备安全要求 第7部分：焊炬） |
| 标记 | 焊炬上应清晰并永久地标出以下信息： |
|  | a. 制造厂给定的型号 |
|  | b. 手柄标准号，以确定枪柄符合本标准要求 |
| 绝缘电阻 | 焊接回路对焊炬的绝缘体外表面≥1MΩ； |
|  | 控制回路对焊炬的绝缘体外表面≥1MΩ； |
|  | 控制回路对焊接回路≥2.5MΩ； |
| 介电强度 | 控制回路对焊炬绝缘体外表面应能承受1000V的高频交流电压,历时60s,无闪络或击穿现象 |
|  | 焊接回路对焊炬的绝缘体表面应能承受3000V的高频交流电压,历时2S,无闪络或击穿现象 |
|  | 控制回路对焊接回路应能承受3000V的高频交流电压,历时2S,无闪络或击穿现象 |
| 温升 | 电缆软管组件的外表面任何一处的温升不应超过40K |
|  | 焊炬（枪）手柄手持位置的外表任何一处温升不超过30K |
|  | 试验后，焊炬（枪）的安全性和操作性能不应降低 |
| 热额定性能 | 焊炬（枪）的负载持续率35% |